# Software Toolchain Overview

Khoa A. Nguyen

Kasey Flight Lab Team

Author Note

Last Update: Feb 1, 2026

Contents

I.	Programming Tool	3

Programming Language	3

C/C++	3

Build System	3

Arm-none-eabi – Arm GNU Toolchain Compiler	3

Cmake – Build System Generator 	3

Installation: Download from their website https://cmake.org/download/	3

Ninja – Build Executor	4

Debugging Tool	4

OpenOCD - Software	4

STLINK - Hardware	4

Build Process Manual	4

# Programming Tool

## Programming Language

### C/C++

## Build System

### Arm-none-eabi – Arm GNU Toolchain Compiler

Installation: download it from their website https://developer.arm.com/downloads/-/gnu-rm

Windows have two main targets: mingw-w64-i686 for legacy and older pc, mingw-w64-x86_64 for most of modern pc
Recommend download MSI for easier installation. After finish still required to add path variable to the system. The path usually is C:\Program Files\Arm\GNU Toolchain mingw-w64-x86_64-arm-none-eabi\bin

### Cmake – Build System Generator

### Download from their website https://cmake.org/download/

Recommend downloading MSI version for easier installation. When running the installation program, check add path variables for no need to do it manually

### Ninja – Build Executor

Installation: Download it from their website https://github.com/ninja-build/ninja/releases

Extract the file and move the ninja.exe to any folder you want but remember to add path variables. This project’s software expects ninja to put in C:\Windows\ninja-win

## Debugging Tool

### OpenOCD - Software

### STLINK - Hardware

# Build Process Manual

Example with nucleo_h563 project build
First time run or after clear up for a clean build
 cmake -S . -B build_h563 -G Ninja  -DCMAKE_TOOLCHAIN_FILE=cmake/gcc-arm-none-eabi.cmake

Explaination:
-S . – generator build instruction the source directory at where the current terminal at right now
-B – generate in the folder build_h563

-G – specify to use ninja build executator

-DCMAKE_TOOLCHAIN_FILE=cmake/gcc-arm-none-eabi.cmake – specify system to use gcc-arm-none-eabi rather than other compiler if the running computer have other compiler like MYS compiler

Starting the build:
ninja -C build_h563
building the project in build_h563